Behavioral Rules, Mapping, and SHACL Shapes Catalog for the implemented use case.

Contents

1. Scope of the document	1

2. Method	1

3. Representation Basis	2

3.1 Reused ontology terms	2

3.2 Behavioral Interoperability Testing Ontology (BITO) terms used for evidence-side fields	3

4. Mapping Basis	4

4.1 Mapping of scenario input dataset items	4

4.2 Mapping of observed evidence items	7

5. Rule Classification	11

5.1 Semantic validation rules	11

5.2 Behavioral validation rules	11

6. Scenario 1 – Announcement of Plan	12

7. Scenario 2 – Shift Preferred Power Sequence	18

8. Scenario 3 – Select Alternative Power Sequence	29

9. Conclusion	48

# 1. Scope of the document

This document defines the behavioral rule catalog, ontology mapping basis, and SHACL / SHACL-SPARQL validation constraints for the Flexible Start use case behavioral validation. The catalog covers Scenarios 1, 2, and 3 of the current Flexible Start validation scope. Scenarios 1 and 2 are included to make the validation basis complete across the selected Flexible Start behaviors. Scenario 3, Select Alternative Power Sequence, is the main implemented demonstration scenario used to validate the ontology-driven behavioral interoperability testing workflow.

# 2. Method

The mapping and rule catalog were prepared using the following method.

- Extract the scenario rules from the current Flexible Start validation scope and keep only Scenarios 1, 2, and 3.

- Reuse SAREF and SAREF4ENER for device, plan, alternatives group, power sequence, slot, state, and timing concepts where a safe direct match exists.

- Reuse PROV-O and OWL-Time for activity, entity, agent, and temporal representation where applicable.

- Use bito: terms only where the required concept is specific to behavioral interoperability testing, validation traceability, evidence organization, request-side information, scenario representation, observed execution, or RDF evidence linking, and where no sufficiently aligned reused ontology term exists.

- Preserve the modeling structure declared in BITO when linking scenarios, observed executions, observed events, trace evidence, and observed state changes. In particular, scenario-level concepts, execution-level evidence, and request-side event fields are not flattened into a single resource unless explicitly required by an implementation-specific validation pattern.

- Use SHACL Core for structural, datatype, cardinality, and direct-value checks.

- Use SHACL-SPARQL when counting, cross-field comparison, ordering, state-transition comparison, or timing comparison is required.

# 3. Representation Basis

The validation data graph is built from three main inputs: the scenario input dataset, the structured observed evidence, and the ontology-based mapping resources. The scenario input dataset mainly contributes the announced flexible-start plan, device information, alternatives group, power sequences, slot information, state declarations, and timing constraints. The observed evidence contributes the post-processed request, acceptance, plan-update, state-change, and execution information used to check whether the expected behavior was actually followed.

The SHACL shape graph is separate from the validation data graph. It contains the SHACL and SHACL-SPARQL constraints used to validate the RDF data graph generated from the scenario input and observed execution evidence. This distinction is important because the data graph represents the scenario and evidence, while the shape graph represents the validation logic.

## 3.1 Reused ontology terms

The following reused ontology terms provide the main representation basis for the current validation scope.

SAREF is reused for the device and generic data representation layer, including saref:Device, saref:Appliance, saref:hasIdentifier, saref:hasState, saref:consistsOf, saref:PropertyValue, saref:hasValue, saref:isMeasuredIn, saref:UnitOfMeasure, saref:PowerUnit, and saref:Watt.

SAREF4ENER is reused for the flexible-start energy profile and power-sequence layer, including s4ener:PowerProfile, s4ener:AlternativesGroup, s4ener:PowerSequence, s4ener:Slot, s4ener:PowerSequenceState, s4ener:Scheduled, s4ener:Inactive, s4ener:Completed, s4ener:hasStartTime, s4ener:hasEndTime, s4ener:hasEarliestStartTime, s4ener:hasLatestEndTime, s4ener:belongsTo, s4ener:nodeRemoteControllable, s4ener:supportsReselection, s4ener:sequenceRemoteControllable, s4ener:isPausable, s4ener:isStoppable, and s4ener:hasDefaultDuration.

PROV-O is reused for generic activity, entity, and agent grounding, including prov:Activity, prov:Entity, prov:Agent, prov:startedAtTime, and prov:endedAtTime, where applicable.

OWL-Time is reused where explicit temporal resources are needed, especially through time:Instant and time:Interval. In the current validation graph, many timing values are represented directly as datatype values, while OWL-Time remains available for more explicit temporal modeling when required.

Terms such as textual descriptions, value sources, and slot value structures should only be included when they are explicitly declared in the ontology resources or safely introduced as implementation-specific extensions. They should not be treated as core reused terms unless they are present in the ontology or imported vocabulary used by the validation graph.

## 3.2 Behavioral Interoperability Testing Ontology (BITO) terms used for evidence-side fields

The Behavioral Interoperability Testing Ontology introduces BITO terms where the observed JSON evidence, validation traceability, scenario representation, request-side information, or RDF evidence-linking structure requires concepts that are not sufficiently covered by SAREF, SAREF4ENER, PROV-O, or OWL-Time.

In the current scope, BITO mainly covers the use case, scenario, implementation under test, interacting actor system, observed event, observed execution, trace evidence, observed state change, request timestamp, requested power sequence, requested start time, acceptance information, observed plan-update timing, observed execution timing, observed plan status, observed run status, and explicit observed sequence-reference roles.

The main scenario-level structure is represented as follows. A bito:Scenario can be linked to the implementation under test through bito:hasIUT, to the external interacting actor system through bito:hasInteractingActorSystem, to the triggering request or interaction through bito:hasTriggeringInteraction, and to the observed execution context through bito:hasObservedExecution.

The observed execution context is then used to organize the evidence. A bito:ObservedExecution can be linked to the observed request or interaction through bito:hasObservedEvent, to the structured evidence package through bito:hasTraceEvidence, and to observed state transitions through bito:hasObservedStateChange.

Request-side fields are represented on bito:ObservedEvent. This includes bito:observedInteractionKind, bito:interactionEndpoint, bito:requestTimestamp, bito:requestedPowerSequence, bito:requestedStartTime, bito:acceptanceOutcome, and bito:acceptedAt.

Execution-side and evidence-side fields are represented through bito:ObservedExecution, bito:TraceEvidence, and bito:ObservedStateChange. Execution timing is attached to the relevant behavioral evidence resource using bito:observedExecutionStart and bito:observedExecutionEnd. Plan and run outcomes can be represented using bito:observedPlanStatus and bito:observedRunStatus. Sequence-role evidence can be represented using properties such as bito:observedPreferredPowerSequence, bito:observedOptionalPowerSequence, bito:observedScheduledPowerSequence, bito:observedSelectedPowerSequence, and bito:observedFormerPreferredPowerSequence.

For state transitions, the preferred ontology-aligned pattern is to use bito:ObservedStateChange, linked to the relevant power sequence through bito:hasRelevantPowerSequence and to the observed source and target states through bito:hasObservedSourceState and bito:hasObservedTargetState. This avoids assigning contradictory states directly to the same s4ener:PowerSequence individual when the graph needs to represent both the state before and after an accepted behavioral request.

# 4. Mapping Basis

The mapping tables below show how the scenario input dataset and observed evidence fields are represented using the ontology terms available in the current validation ontology. The mapping distinguishes direct reuse of SAREF and SAREF4ENER terms, generic reuse of PROV-O and OWL-Time terms, and BITO-specific additions for behavioral interoperability testing, traceability, observed evidence, request-side information, and validation-oriented evidence organization.

Legend: Exact = direct semantic match with an available ontology term; Generic = safe broader grounding using an available ontology term; Pattern = representation through an ontology-supported modeling pattern; BITO = term introduced by the Behavioral Interoperability Testing Ontology.

Some fields, such as entityId, entityType, useCaseName, useCaseVersion, and evidenceMode, are retained as traceability or evidence-organization properties. They are included only where needed for validation-input representation and should not be interpreted as core flexible-start energy-domain concepts.

## 4.1 Mapping of scenario input dataset items

| Dataset item | Meaning / information | Reference ontology term(s) | Ontology term(s) link | Mapping type | Notes |
| --- | --- | --- | --- | --- | --- |
| deviceId | Implementation identifier of the dishwasher device / IUT | saref:Device, saref:Appliance, saref:hasIdentifier | https://saref.etsi.org/core/Device; https://saref.etsi.org/core/Appliance; https://saref.etsi.org/core/hasIdentifier | Generic / Pattern | Represent the dishwasher as a saref:Appliance, which is also a saref:Device. Attach the implementation identifier using saref:hasIdentifier. |
| entityId | Implementation-side entity identifier | bito:entityId | https://w3id.org/bito#entityId | BITO | Used because the dataset distinguishes entityId from the device identifier. |
| entityType = Dishwasher | Dataset-side appliance type | saref:Appliance, bito:entityType | https://saref.etsi.org/core/Appliance; https://w3id.org/bito#entityType | Generic / BITO | Type the IUT as saref:Appliance and preserve the dataset value through bito:entityType. |
| actor = SmartAppliance | Appliance role in the scenario input | saref:Device, saref:Appliance, bito:hasIUT | https://saref.etsi.org/core/Device; https://saref.etsi.org/core/Appliance; https://w3id.org/bito#hasIUT | Generic / BITO | The smart appliance should be treated as the implementation under test. Link the scenario to the appliance using bito:hasIUT. |
| useCase = flexibleStartForWhiteGoods | Use-case name in the dataset | bito:UseCase, bito:useCaseName | https://w3id.org/bito#UseCase; https://w3id.org/bito#useCaseName | BITO | Represent the use case as a bito:UseCase individual and attach the dataset value using bito:useCaseName. |
| usecaseVersion | Version of the use case | bito:useCaseVersion | https://w3id.org/bito#useCaseVersion | BITO | Use as traceability metadata for the use-case version. |
| scenarioSupport | List of supported scenario numbers for the use case | bito:UseCase, bito:hasScenario, bito:Scenario, bito:scenarioNumber | https://w3id.org/bito#UseCase; https://w3id.org/bito#hasScenario; https://w3id.org/bito#Scenario; https://w3id.org/bito#scenarioNumber | BITO / Pattern | Represent the use case as a bito:UseCase individual. For each supported scenario, create a bito:Scenario individual, link it from the use case using bito:hasScenario, and attach the scenario number using bito:scenarioNumber. |
| powerSequenceNode | Node-level flexible-start profile | s4ener:PowerProfile | https://saref.etsi.org/saref4ener/PowerProfile | Exact | Direct SAREF4ENER class for the flexible-start power profile. |
| nodeRemoteControllable | Remote-control declaration at profile level | s4ener:nodeRemoteControllable | https://saref.etsi.org/saref4ener/nodeRemoteControllable | Exact | Direct SAREF4ENER datatype property on s4ener:PowerProfile. |
| supportsReselection | Whether reselection is supported | s4ener:supportsReselection | https://saref.etsi.org/saref4ener/supportsReselection | Exact | Direct SAREF4ENER datatype property on s4ener:PowerProfile. |
| powerSequenceAlternativesRelation | Grouping relation between the power profile, alternatives group, and alternative power sequences | s4ener:PowerProfile, s4ener:AlternativesGroup, s4ener:PowerSequence, saref:consistsOf, s4ener:belongsTo | https://saref.etsi.org/saref4ener/PowerProfile; https://saref.etsi.org/saref4ener/AlternativesGroup; https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/core/consistsOf; https://saref.etsi.org/saref4ener/belongsTo | Exact / Pattern | Represent the profile as s4ener:PowerProfile and the alternatives group as s4ener:AlternativesGroup. Link the profile to the alternatives group using saref:consistsOf. Link the alternatives group to its power sequences using saref:consistsOf. Optionally link each sequence back to the alternatives group using s4ener:belongsTo. |
| alternativesId | Identifier of the alternatives group | s4ener:AlternativesGroup, saref:hasIdentifier | https://saref.etsi.org/saref4ener/AlternativesGroup; https://saref.etsi.org/core/hasIdentifier | Pattern | Attach the alternatives group identifier to the s4ener:AlternativesGroup individual using saref:hasIdentifier. |
| powerSequence | Selectable execution plan or sequence | s4ener:PowerSequence | https://saref.etsi.org/saref4ener/PowerSequence | Exact | Direct SAREF4ENER class for a power sequence. |
| sequenceId | Identifier of a power sequence | s4ener:PowerSequence, saref:hasIdentifier | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/core/hasIdentifier | Pattern | Attach the sequence identifier to the s4ener:PowerSequence individual using saref:hasIdentifier. |
| powerUnit = W | Unit of the slot power value | saref:UnitOfMeasure, saref:PowerUnit, saref:Watt, saref:isMeasuredIn | https://saref.etsi.org/core/UnitOfMeasure; https://saref.etsi.org/core/PowerUnit; https://saref.etsi.org/core/Watt; https://saref.etsi.org/core/isMeasuredIn | Pattern | Represent watt as the available unit individual saref:Watt, typed as a power unit, and link the property value to it using saref:isMeasuredIn. |
| state | Current power-sequence state | saref:hasState, s4ener:PowerSequenceState, s4ener:Scheduled, s4ener:Inactive, s4ener:Completed | https://saref.etsi.org/core/hasState; https://saref.etsi.org/saref4ener/PowerSequenceState; https://saref.etsi.org/saref4ener/Scheduled; https://saref.etsi.org/saref4ener/Inactive; https://saref.etsi.org/saref4ener/Completed | Exact | Link each power sequence to its state using saref:hasState. Use the available SAREF4ENER state individuals. |
| startTime | Configured or announced sequence start time | s4ener:hasStartTime | https://saref.etsi.org/saref4ener/hasStartTime | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| endTime | Configured or announced sequence end time | s4ener:hasEndTime | https://saref.etsi.org/saref4ener/hasEndTime | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| earliestStartTime | Earliest allowed sequence start time | s4ener:hasEarliestStartTime | https://saref.etsi.org/saref4ener/hasEarliestStartTime | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| latestEndTime | Latest allowed sequence end time | s4ener:hasLatestEndTime | https://saref.etsi.org/saref4ener/hasLatestEndTime | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| isPausable | Whether the sequence can be paused | s4ener:isPausable | https://saref.etsi.org/saref4ener/isPausable | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| isStoppable | Whether the sequence can be stopped | s4ener:isStoppable | https://saref.etsi.org/saref4ener/isStoppable | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| sequenceRemoteControllable | Whether the sequence can be remotely controlled | s4ener:sequenceRemoteControllable | https://saref.etsi.org/saref4ener/sequenceRemoteControllable | Exact | Direct SAREF4ENER datatype property on s4ener:PowerSequence. |
| powerTimeSlot | One slot of a power sequence | s4ener:Slot, saref:consistsOf | https://saref.etsi.org/saref4ener/Slot; https://saref.etsi.org/core/consistsOf | Exact / Pattern | Represent each slot as s4ener:Slot and link the containing power sequence to the slot using saref:consistsOf. |
| power value | Numeric power value associated with a slot value representation | saref:PropertyValue, saref:hasValue, saref:isMeasuredIn, saref:PowerUnit, saref:Watt | https://saref.etsi.org/core/PropertyValue; https://saref.etsi.org/core/hasValue; https://saref.etsi.org/core/isMeasuredIn; https://saref.etsi.org/core/PowerUnit; https://saref.etsi.org/core/Watt | Pattern | Represent the numeric value as a saref:PropertyValue with saref:hasValue and saref:isMeasuredIn saref:Watt. |
| defaultDuration | Default slot duration | s4ener:hasDefaultDuration | https://saref.etsi.org/saref4ener/hasDefaultDuration | Exact | Direct SAREF4ENER datatype property on s4ener:Slot. |
| POST request body sequenceId/startTime | Requested shift or alternative selection in Scenarios 2 and 3 | bito:ObservedEvent, bito:requestedPowerSequence, bito:requestedStartTime | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#requestedPowerSequence; https://w3id.org/bito#requestedStartTime | BITO / Pattern | Represent request-side fields on a bito:ObservedEvent. Link the request to the target sequence using bito:requestedPowerSequence and attach the requested start time using bito:requestedStartTime. |

## 4.2 Mapping of observed evidence items

| Observed evidence item | Meaning / information | Reference ontology term(s) | Ontology term(s) link | Mapping type | Notes |
| --- | --- | --- | --- | --- | --- |
| scenario / scenarioName | Scenario number and scenario label in the evidence | bito:Scenario, bito:scenarioNumber, bito:scenarioName | https://w3id.org/bito#Scenario; https://w3id.org/bito#scenarioNumber; https://w3id.org/bito#scenarioName | BITO | Represent the observed scenario as a bito:Scenario individual. Attach the scenario number using bito:scenarioNumber and the scenario label using bito:scenarioName. |
| observedInteractionKind | Type of observed interaction | bito:ObservedEvent, bito:observedInteractionKind | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#observedInteractionKind | BITO | Attach the interaction kind to the bito:ObservedEvent. This value scopes rules such as plan observation, preferred-sequence shift request, or alternative-sequence selection request. |
| evidenceMode | Mode used to organize evidence sessions | bito:TraceEvidence, bito:evidenceMode | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#evidenceMode | BITO | Attach the evidence mode to the bito:TraceEvidence resource. This field supports evidence organization. |
| interactingActorSystem = CCM | External actor or system interacting with the IUT | prov:Agent, bito:hasInteractingActorSystem | http://www.w3.org/ns/prov#Agent; https://w3id.org/bito#hasInteractingActorSystem | Generic / BITO | Type CCM as prov:Agent. Link the bito:Scenario to CCM using bito:hasInteractingActorSystem. |
| interactionEndpoint | Observed API endpoint | bito:ObservedEvent, bito:interactionEndpoint | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#interactionEndpoint | BITO | Attach the endpoint string to the bito:ObservedEvent. |
| observedAt | Observation timestamp | bito:BehavioralElement, bito:observedAt | https://w3id.org/bito#BehavioralElement; https://w3id.org/bito#observedAt | BITO | Attach the observation timestamp to the relevant bito:BehavioralElement used for the observed evidence item. |
| requestTimestamp | Timestamp of the request activity | bito:ObservedEvent, bito:requestTimestamp | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#requestTimestamp | BITO | Attach the request timestamp to the bito:ObservedEvent. |
| requestedSequenceId | Requested target power sequence | bito:ObservedEvent, bito:requestedPowerSequence, s4ener:PowerSequence, saref:hasIdentifier | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#requestedPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/core/hasIdentifier | BITO / Pattern | Link the observed request event to the requested s4ener:PowerSequence using bito:requestedPowerSequence. Preserve the sequence identifier on the sequence individual using saref:hasIdentifier. |
| requestedStartTime | Requested new sequence start time | bito:ObservedEvent, bito:requestedStartTime | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#requestedStartTime | BITO | Attach the requested start time to the bito:ObservedEvent. |
| acceptanceOutcome | Observed acceptance result | bito:ObservedEvent, bito:acceptanceOutcome | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#acceptanceOutcome | BITO | Attach the acceptance result to the bito:ObservedEvent. |
| acceptedAt | Timestamp at which the request was accepted | bito:ObservedEvent, bito:acceptedAt | https://w3id.org/bito#ObservedEvent; https://w3id.org/bito#acceptedAt | BITO | Attach the acceptance timestamp to the bito:ObservedEvent. |
| observedPreferredSequenceId | Observed preferred sequence in Scenario 1 | bito:TraceEvidence, bito:observedPreferredPowerSequence, s4ener:PowerSequence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedPreferredPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence | BITO | Attach the observed preferred sequence reference to the bito:TraceEvidence resource. |
| observedOptionalSequenceId | Observed optional sequence in Scenario 1 | bito:TraceEvidence, bito:observedOptionalPowerSequence, s4ener:PowerSequence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedOptionalPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence | BITO | Attach the observed optional sequence reference to the bito:TraceEvidence resource. |
| observedScheduledSequenceId | Observed scheduled sequence after accepted shift | bito:TraceEvidence, bito:observedScheduledPowerSequence, s4ener:PowerSequence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedScheduledPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence | BITO | Attach the observed scheduled sequence reference to the bito:TraceEvidence resource. |
| observedSelectedSequenceId | Observed selected sequence after accepted selection | bito:TraceEvidence, bito:observedSelectedPowerSequence, s4ener:PowerSequence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedSelectedPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence | BITO | Attach the observed selected sequence reference to the bito:TraceEvidence resource. |
| observedFormerPreferredSequenceId | Observed former preferred sequence after accepted selection | bito:TraceEvidence, bito:observedFormerPreferredPowerSequence, s4ener:PowerSequence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedFormerPreferredPowerSequence; https://saref.etsi.org/saref4ener/PowerSequence | BITO | Attach the observed former preferred sequence reference to the bito:TraceEvidence resource. |
| observedPreferredSequenceState / observedOptionalSequenceState / observedScheduledSequenceState / observedSelectedSequenceState / observedFormerPreferredSequenceState | Observed sequence state values | s4ener:PowerSequence, saref:hasState, s4ener:PowerSequenceState, s4ener:Scheduled, s4ener:Inactive, s4ener:Completed, bito:ObservedStateChange, bito:hasRelevantPowerSequence, bito:hasObservedSourceState, bito:hasObservedTargetState | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/core/hasState; https://saref.etsi.org/saref4ener/PowerSequenceState; https://saref.etsi.org/saref4ener/Scheduled; https://saref.etsi.org/saref4ener/Inactive; https://saref.etsi.org/saref4ener/Completed; https://w3id.org/bito#ObservedStateChange; https://w3id.org/bito#hasRelevantPowerSequence; https://w3id.org/bito#hasObservedSourceState; https://w3id.org/bito#hasObservedTargetState | Exact / BITO / Pattern | For a stable observed sequence state, link the relevant s4ener:PowerSequence to its state using saref:hasState. For before/after behavior, represent the transition as bito:ObservedStateChange, linked to the power sequence through bito:hasRelevantPowerSequence and to the source and target states through bito:hasObservedSourceState and bito:hasObservedTargetState. |
| observedPreferredSequenceStartTime / observedUpdatedStartTime | Observed or updated sequence start time | s4ener:PowerSequence, s4ener:hasStartTime | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/saref4ener/hasStartTime | Pattern | Attach the observed or updated start time to the relevant observed s4ener:PowerSequence resource in the validation data graph. |
| observedPreferredSequenceEndTime / observedUpdatedEndTime | Observed or updated sequence end time | s4ener:PowerSequence, s4ener:hasEndTime | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/saref4ener/hasEndTime | Pattern | Attach the observed or updated end time to the relevant observed s4ener:PowerSequence resource in the validation data graph. |
| observedPreferredSequenceEarliestStartTime / observedOptionalSequenceEarliestStartTime | Observed earliest allowed start time | s4ener:PowerSequence, s4ener:hasEarliestStartTime | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/saref4ener/hasEarliestStartTime | Pattern | Use when the observed evidence preserves the sequence timing bounds. |
| observedPreferredSequenceLatestEndTime / observedOptionalSequenceLatestEndTime | Observed latest allowed end time | s4ener:PowerSequence, s4ener:hasLatestEndTime | https://saref.etsi.org/saref4ener/PowerSequence; https://saref.etsi.org/saref4ener/hasLatestEndTime | Pattern | Use when the observed evidence preserves the sequence timing bounds. |
| observedPlanUpdatedAt | Timestamp of the observed plan update | bito:BehavioralElement, bito:observedPlanUpdatedAt | https://w3id.org/bito#BehavioralElement; https://w3id.org/bito#observedPlanUpdatedAt | BITO | Attach the plan-update timestamp to the relevant bito:BehavioralElement used for the observed plan update. |
| observedExecutionStart | Observed execution start time | bito:BehavioralElement, bito:observedExecutionStart | https://w3id.org/bito#BehavioralElement; https://w3id.org/bito#observedExecutionStart | BITO | Attach the execution start time to the relevant bito:BehavioralElement used for execution timing evidence. |
| observedExecutionEnd | Observed execution end time | bito:BehavioralElement, bito:observedExecutionEnd | https://w3id.org/bito#BehavioralElement; https://w3id.org/bito#observedExecutionEnd | BITO | Attach the execution end time to the relevant bito:BehavioralElement used for execution timing evidence. |
| observedPlanStatus | Observed plan-level status | bito:TraceEvidence, bito:observedPlanStatus | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedPlanStatus | BITO | Attach the observed plan status to the bito:TraceEvidence resource. This is separate from the validation verdict. |
| observedRunStatus | Observed run status | bito:TraceEvidence, bito:observedRunStatus | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#observedRunStatus | BITO | Attach the observed run status to the bito:TraceEvidence resource. This value supports completion checks in Scenarios 2 and 3. |
| Evidence session / evidence record | Structured evidence package for one scenario execution | bito:TraceEvidence, bito:hasTraceEvidence | https://w3id.org/bito#TraceEvidence; https://w3id.org/bito#hasTraceEvidence | BITO | Represent the evidence package as bito:TraceEvidence. Link it from bito:ObservedExecution using bito:hasTraceEvidence. |
| Observed request / observed interaction | Request or interaction observed in the evidence | bito:ObservedEvent, prov:Activity | https://w3id.org/bito#ObservedEvent; http://www.w3.org/ns/prov#Activity | BITO / Generic | Type the observed request or interaction as bito:ObservedEvent. This class is grounded as a prov:Activity. |
| Observed execution / appliance run | Execution context after the accepted request or plan update | bito:ObservedExecution, prov:Activity | https://w3id.org/bito#ObservedExecution; http://www.w3.org/ns/prov#Activity | BITO / Generic | Type the execution context as bito:ObservedExecution. This resource organizes the observed event, trace evidence, and state-change evidence. |
| Observed state change | Evidence that a sequence changed state after request acceptance | bito:ObservedStateChange, bito:hasRelevantPowerSequence, bito:hasObservedSourceState, bito:hasObservedTargetState | https://w3id.org/bito#ObservedStateChange; https://w3id.org/bito#hasRelevantPowerSequence; https://w3id.org/bito#hasObservedSourceState; https://w3id.org/bito#hasObservedTargetState | BITO / Pattern | Use this pattern for explicit state transitions, especially when a selected sequence becomes scheduled or a former preferred sequence becomes inactive. |
| Scenario to observed execution link | Link from the scenario to the observed execution context | bito:Scenario, bito:hasObservedExecution, bito:ObservedExecution | https://w3id.org/bito#Scenario; https://w3id.org/bito#hasObservedExecution; https://w3id.org/bito#ObservedExecution | BITO | Use bito:hasObservedExecution from bito:Scenario to bito:ObservedExecution. |
| Scenario to triggering interaction link | Link from the scenario to the triggering request or interaction | bito:Scenario, bito:hasTriggeringInteraction, bito:ObservedEvent | https://w3id.org/bito#Scenario; https://w3id.org/bito#hasTriggeringInteraction; https://w3id.org/bito#ObservedEvent | BITO | Use bito:hasTriggeringInteraction from bito:Scenario to bito:ObservedEvent. |
| Execution to observed event link | Link from observed execution context to the observed request event | bito:ObservedExecution, bito:hasObservedEvent, bito:ObservedEvent | https://w3id.org/bito#ObservedExecution; https://w3id.org/bito#hasObservedEvent; https://w3id.org/bito#ObservedEvent | BITO | Use bito:hasObservedEvent from bito:ObservedExecution to bito:ObservedEvent. |
| Execution to trace evidence link | Link from observed execution context to the structured evidence package | bito:ObservedExecution, bito:hasTraceEvidence, bito:TraceEvidence | https://w3id.org/bito#ObservedExecution; https://w3id.org/bito#hasTraceEvidence; https://w3id.org/bito#TraceEvidence | BITO | Use bito:hasTraceEvidence from bito:ObservedExecution to bito:TraceEvidence. |
| Execution to observed state-change link | Link from observed execution context to state-transition evidence | bito:ObservedExecution, bito:hasObservedStateChange, bito:ObservedStateChange | https://w3id.org/bito#ObservedExecution; https://w3id.org/bito#hasObservedStateChange; https://w3id.org/bito#ObservedStateChange | BITO | Use bito:hasObservedStateChange from bito:ObservedExecution to bito:ObservedStateChange. |

# 5. Rule Classification

The current validation scope contains two rule groups. Semantic rules check whether the announced plan and evidence are structurally and semantically well formed. Behavioral rules check whether the observed execution follows the scenario behavior and achieves the expected operational outcome.

## 5.1 Semantic validation rules

| Rule ID | Scenario | Rule statement | Rule type | Validation method |
| --- | --- | --- | --- | --- |
| S1-R1 | Scenario 1 | Power profile declares nodeRemoteControllable | Structural / semantic | SHACL |
| S1-R2 | Scenario 1 | Power profile declares supportsReselection | Structural / semantic | SHACL |
| S1-R3 | Scenario 1 | Exactly one power sequence is scheduled in the alternatives group | State / counting | SHACL-SPARQL |
| S1-R4 | Scenario 1 | Inactive optional sequence has no start time | State / semantic | SHACL-SPARQL |
| S1-R5 | Scenario 1 | Scheduled sequence has required timing fields | Structural / semantic | SHACL-SPARQL |
| S1-R6 | Scenario 1 | End time is after start time | Timing | SHACL-SPARQL |
| S1-R7 | Scenario 1 | Start time is not before earliest start time | Timing | SHACL-SPARQL |
| S1-R8 | Scenario 1 | Each power sequence has at least one slot and each slot has default duration | Structural | SHACL |

## 5.2 Behavioral validation rules

| Rule ID | Scenario | Rule statement | Rule type | Validation method |
| --- | --- | --- | --- | --- |
| S2-R1 | Scenario 2 | Shift request targets the currently scheduled preferred sequence | Trigger | SHACL-SPARQL |
| S2-R2 | Scenario 2 | Requested start time is within the allowed lower bound | Timing | SHACL-SPARQL |
| S2-R3 | Scenario 2 | Acceptance outcome is present and valid | Outcome / evidence | SHACL-SPARQL |
| S2-R4 | Scenario 2 | After acceptance, the same sequence remains scheduled | State transition | SHACL-SPARQL |
| S2-R5 | Scenario 2 | Updated start time equals requested start time | Timing / transition | SHACL-SPARQL |
| S2-R6 | Scenario 2 | Observed execution start shall not occur before the updated planned start | Timing / ordering | SHACL-SPARQL |
| S2-R7 | Scenario 2 | Observed run status is COMPLETED | Outcome | SHACL-SPARQL |
| S3-R1 | Scenario 3 | Selection request targets an inactive optional sequence | Trigger | SHACL-SPARQL |
| S3-R2 | Scenario 3 | Requested start time satisfies the allowed time window | Timing | SHACL-SPARQL |
| S3-R3 | Scenario 3 | Remote-control preconditions hold for the profile and selected sequence | Precondition / semantic | SHACL-SPARQL |
| S3-R4 | Scenario 3 | If reselection is false, a second accepted selection is not allowed | Counting | SHACL-SPARQL |
| S3-G1 | Scenario 3 | Acceptance gate: observed selection request is accepted | Gate / evidence | SHACL-SPARQL |
| S3-R5 | Scenario 3 | Selected sequence becomes scheduled after acceptance | State transition | SHACL-SPARQL |
| S3-R6 | Scenario 3 | Former preferred sequence becomes inactive after acceptance | State transition | SHACL-SPARQL |
| S3-R7 | Scenario 3 | Observed execution start shall not occur before the updated planned start | Timing / ordering | SHACL-SPARQL |
| S3-R8 | Scenario 3 | Observed run status is COMPLETED | Outcome | SHACL-SPARQL |

S3-G1 is included as an implementation gate for Scenario 3. It is not an additional behavioral rule. It represents the acceptance check that must pass before the Scenario 3 state-transition rules S3-R5 and S3-R6 are evaluated. The validation method column reflects the implementation used in the SHACL shapes. Some checks could also be expressed with SHACL Core, but they are marked as SHACL-SPARQL where the implemented shape uses SPARQL for missing-evidence handling, cross-resource checking, or stepwise behavioral validation.

# 6. Scenario 1 – Announcement of Plan

Scenario 1 validates the announced flexible-start plan before any behavioral change request is applied. These rules mainly check plan structure, announced states, and announced timing consistency.

S1-R1 – Power profile declares remote controllability

The announced power profile shall explicitly declare nodeRemoteControllable.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix xsd: <http://www.w3.org/2001/XMLSchema#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R1_RemoteControllable

a sh:NodeShape ;

sh:targetClass s4ener:PowerProfile ;

dcterms:identifier "S1-R1" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R1 - Power profile declares remote controllability" ;

sh:property [

sh:path s4ener:nodeRemoteControllable ;

sh:minCount 1 ;

sh:maxCount 1 ;

sh:datatype xsd:boolean ;

sh:message "S1-R1 FAIL: PowerProfile shall declare s4ener:nodeRemoteControllable." ;

] .

This shape is simple. It only checks that the plan explicitly states whether the node is remotely controllable.

S1-R2 – Power profile declares supportsReselection

The announced power profile shall explicitly declare supportsReselection.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix xsd: <http://www.w3.org/2001/XMLSchema#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R2_SupportsReselection

a sh:NodeShape ;

sh:targetClass s4ener:PowerProfile ;

dcterms:identifier "S1-R2" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R2 - Power profile declares supportsReselection" ;

sh:property [

sh:path s4ener:supportsReselection ;

sh:minCount 1 ;

sh:maxCount 1 ;

sh:datatype xsd:boolean ;

sh:message "S1-R2 FAIL: PowerProfile shall declare s4ener:supportsReselection." ;

] .

This shape checks that the plan states whether reselection is supported. Without this, later behavioral interpretation is incomplete.

S1-R3 – Exactly one scheduled sequence in the alternatives group

The alternatives group shall contain exactly one power sequence in state s4ener:Scheduled.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R3_ExactlyOneScheduledSequence

a sh:NodeShape ;

sh:targetClass s4ener:AlternativesGroup ;

dcterms:identifier "S1-R3" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R3 - Exactly one scheduled sequence in the alternatives group" ;

sh:sparql [

sh:message "S1-R3 FAIL: exactly one PowerSequence shall be in state s4ener:Scheduled." ;

sh:select """

SELECT $this

WHERE {

OPTIONAL {

$this saref:consistsOf ?scheduledSeq .

?scheduledSeq saref:hasState s4ener:Scheduled .

}

}

GROUP BY $this

HAVING (COUNT(DISTINCT ?scheduledSeq) != 1)

""" ;

] .

This rule uses SHACL-SPARQL because it counts how many scheduled power sequences appear in one alternatives group. The query fails when there are zero scheduled sequences or more than one scheduled sequence.

S1-R4 – Inactive optional sequence has no start time

Any optional sequence that is inactive shall not already carry a start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R4_InactiveNoStartTime

a sh:NodeShape ;

sh:targetClass s4ener:PowerSequence ;

dcterms:identifier "S1-R4" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R4 - Inactive optional sequence has no start time" ;

sh:sparql [

sh:message "S1-R4 FAIL: a sequence in state s4ener:Inactive shall not have s4ener:hasStartTime." ;

sh:select """

SELECT $this

WHERE {

$this saref:hasState s4ener:Inactive .

$this s4ener:hasStartTime ?startTime .

}

""" ;

] .

This shape checks that an inactive optional sequence is only announced as available and not already scheduled to start.

S1-R5 – Scheduled sequence has required timing fields

A scheduled sequence shall have start time, earliest start time, and latest end time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R5_ScheduledTimingFields

a sh:NodeShape ;

sh:targetClass s4ener:PowerSequence ;

dcterms:identifier "S1-R5" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R5 - Scheduled sequence has required timing fields" ;

sh:sparql [

sh:message "S1-R5 FAIL: a scheduled PowerSequence shall have startTime, earliestStartTime, and latestEndTime." ;

sh:select """

SELECT $this

WHERE {

$this saref:hasState s4ener:Scheduled .

FILTER (

NOT EXISTS { $this s4ener:hasStartTime ?startTime } ||

NOT EXISTS { $this s4ener:hasEarliestStartTime ?earliestStartTime } ||

NOT EXISTS { $this s4ener:hasLatestEndTime ?latestEndTime }

)

}

""" ;

] .

This rule checks that the announced scheduled sequence contains the timing information needed for later behavioral validation.

S1-R6 – End time is after start time

For any sequence with both start and end time, end time shall be strictly after start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R6_EndAfterStart

a sh:NodeShape ;

sh:targetClass s4ener:PowerSequence ;

dcterms:identifier "S1-R6" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R6 - End time is after start time" ;

sh:sparql [

sh:message "S1-R6 FAIL: endTime shall be strictly greater than startTime." ;

sh:select """

SELECT $this

WHERE {

$this

s4ener:hasStartTime ?startTime ;

s4ener:hasEndTime ?endTime .

FILTER (?endTime <= ?startTime)

}

""" ;

] .

This rule compares two timestamps. That is why SHACL-SPARQL is needed instead of SHACL Core only.

S1-R7 – Start time is not before earliest start time

For a scheduled sequence, start time shall not be earlier than earliest start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R7_StartAfterEarliest

a sh:NodeShape ;

sh:targetClass s4ener:PowerSequence ;

dcterms:identifier "S1-R7" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R7 - Start time is not before earliest start time" ;

sh:sparql [

sh:message "S1-R7 FAIL: startTime shall be greater than or equal to earliestStartTime." ;

sh:select """

SELECT $this

WHERE {

$this

saref:hasState s4ener:Scheduled ;

s4ener:hasStartTime ?startTime ;

s4ener:hasEarliestStartTime ?earliestStartTime .

FILTER (?startTime < ?earliestStartTime)

}

""" ;

] .

This rule confirms that the announced schedule respects the lower timing boundary declared for that sequence.

S1-R8 – Sequence has slots and slot duration

Each power sequence shall contain at least one slot, and each slot shall declare a default duration.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix xsd: <http://www.w3.org/2001/XMLSchema#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S1_R8_SequenceHasSlots

a sh:NodeShape ;

sh:targetClass s4ener:PowerSequence ;

dcterms:identifier "S1-R8" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R8 - Power sequence has at least one slot" ;

sh:property [

sh:path saref:consistsOf ;

sh:class s4ener:Slot ;

sh:minCount 1 ;

sh:message "S1-R8 FAIL: each PowerSequence shall contain at least one Slot." ;

] .

val:Shape_S1_R8_SlotHasDuration

a sh:NodeShape ;

sh:targetClass s4ener:Slot ;

dcterms:identifier "S1-R8" ;

val:validationStep "Scenario1-PlanAnnouncement" ;

rdfs:label "S1-R8 - Slot has default duration" ;

sh:property [

sh:path s4ener:hasDefaultDuration ;

sh:minCount 1 ;

sh:datatype xsd:duration ;

sh:message "S1-R8 FAIL: each Slot shall have s4ener:hasDefaultDuration." ;

] .

These two shapes validate the minimum slot structure needed to compute or compare sequence duration later.

# 7. Scenario 2 – Shift Preferred Power Sequence

Scenario 2 validates a shift request on the currently scheduled preferred sequence. The rule chain checks the request, the acceptance, the updated plan, and the observed execution outcome.

S2-R1 – Shift request targets the scheduled preferred sequence

The request activity shall refer to the sequence that is currently scheduled.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S2_R1_ShiftTargetsScheduled

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R1" ;

val:validationStep "Scenario2-RequestAndPreconditions" ;

rdfs:label "S2-R1 - Shift request targets the scheduled preferred sequence" ;

sh:sparql [

sh:message "S2-R1 INCONCLUSIVE: requestedPowerSequence is missing from the preferred-sequence shift request." ;

sh:select """

SELECT $this

WHERE {

$this bito:observedInteractionKind "preferred sequence shift request" .

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R1 INCONCLUSIVE: state of the requested power sequence is missing." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq saref:hasState ?state .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R1 FAIL: the preferred-sequence shift request shall target a power sequence in state s4ener:Scheduled." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:requestedPowerSequence ?seq .

?seq saref:hasState ?state .

FILTER (?state != s4ener:Scheduled)

}

""" ;

] .

This rule makes sure the shift request is applied to the sequence that is currently the preferred scheduled one.

S2-R2 – Requested start time is within the allowed lower bound

The requested start time shall be greater than or equal to the sequence earliest start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S2_R2_RequestedStartWithinLowerBound

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R2" ;

val:validationStep "Scenario2-RequestAndPreconditions" ;

rdfs:label "S2-R2 - Requested start time is within the allowed lower bound" ;

sh:sparql [

sh:message "S2-R2 INCONCLUSIVE: requestedStartTime is missing from the preferred-sequence shift request." ;

sh:select """

SELECT $this

WHERE {

$this bito:observedInteractionKind "preferred sequence shift request" .

FILTER NOT EXISTS {

$this bito:requestedStartTime ?requestedStartTime .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R2 INCONCLUSIVE: requestedPowerSequence or earliestStartTime is missing." ;

sh:select """

SELECT $this

WHERE {

$this bito:observedInteractionKind "preferred sequence shift request" .

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

?seq s4ener:hasEarliestStartTime ?earliestStartTime .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R2 FAIL: requestedStartTime shall be greater than or equal to earliestStartTime." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:requestedStartTime ?requestedStartTime ;

bito:requestedPowerSequence ?seq .

?seq s4ener:hasEarliestStartTime ?earliestStartTime .

FILTER (?requestedStartTime < ?earliestStartTime)

}

""" ;

] .

This rule first checks that the requested start time exists and then checks it against the allowed lower bound.

S2-R3 – Acceptance outcome is present and valid

The request activity shall record an acceptance outcome value of accepted or rejected.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

val:Shape_S2_R3_AcceptanceOutcome

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R3" ;

val:validationStep "Scenario2-AcceptanceAndUpdatedPlan" ;

rdfs:label "S2-R3 - Acceptance outcome is present and valid" ;

sh:sparql [

sh:message "S2-R3 INCONCLUSIVE: acceptanceOutcome is missing from the preferred-sequence shift request." ;

sh:select """

SELECT $this

WHERE {

$this bito:observedInteractionKind "preferred sequence shift request" .

FILTER NOT EXISTS {

$this bito:acceptanceOutcome ?acceptanceOutcome .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R3 FAIL: acceptanceOutcome shall be accepted or rejected." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome ?acceptanceOutcome .

FILTER (?acceptanceOutcome NOT IN ("accepted", "rejected"))

}

""" ;

] .

This is a direct evidence-quality check. It confirms that the request has a clear observed acceptance result.

S2-R4 – Same sequence remains scheduled after acceptance

After an accepted shift, the same sequence shall remain scheduled in the updated plan evidence.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

@prefix saref: <https://saref.etsi.org/core/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S2_R4_SameSequenceStillScheduled

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R4" ;

val:validationStep "Scenario2-AcceptanceAndUpdatedPlan" ;

rdfs:label "S2-R4 - Same sequence remains scheduled after acceptance" ;

sh:sparql [

sh:message "S2-R4 INCONCLUSIVE: observed execution or trace evidence is missing for the accepted shift request." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R4 FAIL: after an accepted shift, the requested sequence shall remain the scheduled sequence." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

FILTER NOT EXISTS {

?traceEvidence bito:observedScheduledPowerSequence ?seq .

?seq saref:hasState s4ener:Scheduled .

}

}

""" ;

] .

This rule uses the updated evidence. It checks that the same sequence is still the scheduled one after acceptance.

S2-R5 – Updated start time equals requested start time

After an accepted shift, the updated observed start time shall equal the requested start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S2_R5_UpdatedStartEqualsRequested

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R5" ;

val:validationStep "Scenario2-AcceptanceAndUpdatedPlan" ;

rdfs:label "S2-R5 - Updated start time equals requested start time" ;

sh:sparql [

sh:message "S2-R5 INCONCLUSIVE: requestedStartTime or requestedPowerSequence is missing from the accepted shift request." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

$this

bito:requestedStartTime ?requestedStartTime ;

bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R5 INCONCLUSIVE: updated planned start time is missing for the requested sequence." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq s4ener:hasStartTime ?updatedStartTime .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R5 FAIL: updated planned start time shall equal requestedStartTime." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" ;

bito:requestedStartTime ?requestedStartTime ;

bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq s4ener:hasStartTime ?requestedStartTime .

}

}

""" ;

] .

This rule compares the requested shift with the updated plan evidence to confirm that the accepted request was correctly reflected. The updated planned start time is checked through s4ener:hasStartTime on the requested sequence because bito:observedUpdatedStartTime is not part of the ontology.

S2-R6 – Observed execution start shall not occur before the updated planned start

Observed execution start shall not occur before the updated planned start. Any tolerance-based lateness check shall be applied through implementation-specific temporal comparison logic where required.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

val:Shape_S2_R6_ExecutionStartNotBeforeUpdatedPlan

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R6" ;

val:validationStep "Scenario2-ExecutionTimingAndOutcome" ;

rdfs:label "S2-R6 - Execution start is not before updated planned start" ;

sh:sparql [

sh:message "S2-R6 INCONCLUSIVE: observed execution is missing for the accepted shift request." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution bito:hasObservedEvent $this .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R6 INCONCLUSIVE: observed execution start time is missing." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

?observedExecution bito:hasObservedEvent $this .

FILTER NOT EXISTS {

?observedExecution bito:observedExecutionStart ?actualExecutionStart .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R6 INCONCLUSIVE: updated planned start time is missing for the requested sequence." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq s4ener:hasStartTime ?updatedPlannedStart .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R6 FAIL: observed execution start shall not be earlier than the updated planned start time." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

?observedExecution

bito:hasObservedEvent $this ;

bito:observedExecutionStart ?actualExecutionStart .

?seq s4ener:hasStartTime ?updatedPlannedStart .

FILTER (?actualExecutionStart < ?updatedPlannedStart)

}

""" ;

] .

This rule connects the updated plan evidence with the observed execution activity. It checks whether execution really follows the accepted plan.

S2-R7 – Observed run status is COMPLETED

The observed run status shall be COMPLETED.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

@prefix bito: <https://w3id.org/bito#> .

val:Shape_S2_R7_RunStatusCompleted

a sh:NodeShape ;

sh:targetClass bito:ObservedEvent ;

dcterms:identifier "S2-R7" ;

val:validationStep "Scenario2-ExecutionTimingAndOutcome" ;

rdfs:label "S2-R7 - Observed run status is COMPLETED" ;

sh:sparql [

sh:message "S2-R7 INCONCLUSIVE: observed execution or trace evidence is missing for the accepted shift request." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R7 INCONCLUSIVE: observedRunStatus is missing from trace evidence." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

FILTER NOT EXISTS {

?traceEvidence bito:observedRunStatus ?runStatus .

}

}

""" ;

] ;

sh:sparql [

sh:message "S2-R7 FAIL: observedRunStatus shall be COMPLETED." ;

sh:select """

SELECT $this

WHERE {

$this

bito:observedInteractionKind "preferred sequence shift request" ;

bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

?traceEvidence bito:observedRunStatus ?runStatus .

FILTER (?runStatus != "COMPLETED")

}

""" ;

] .

This is the final outcome check for Scenario 2. It confirms that the shifted sequence actually completed.

# 8. Scenario 3 – Select Alternative Power Sequence

Scenario 3 validates the selection of an alternative sequence. The rule chain checks the request, the timing window, the remote-control preconditions, the accepted state changes, and the observed execution outcome. In this scenario, the selected alternative power sequence is sequence 45, and the former preferred sequence is sequence 44.

S3-R1 – Selection request targets an inactive optional sequence

The selected alternative power sequence, here sequence 45, must be initially inactive before the accepted selection request is applied.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R1_SelectedSequenceInitiallyInactive

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R1" ;

val:validationStep "Step1-InitialStateAndPreconditions" ;

rdfs:label "S3-R1 - Selected sequence is initially inactive" ;

sh:sparql [

sh:message "S3-R1 INCONCLUSIVE: requestedPowerSequence is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R1 FAIL: Scenario 3 request must target alternative sequence 45." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER (?seq != <https://w3id.org/bito/example/flexible-start/power-sequence/45>)

}

""" ;

] ;

sh:sparql [

sh:message "S3-R1 INCONCLUSIVE: observed state-change evidence is missing for the selected sequence." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasObservedStateChange ?stateChange .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?seq ;

bito:hasObservedSourceState ?sourceState .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R1 FAIL: selected sequence 45 must be initially Inactive." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasObservedStateChange ?stateChange .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?seq ;

bito:hasObservedSourceState ?sourceState .

FILTER (?sourceState != s4ener:Inactive)

}

""" ;

] .

This rule checks two points. First, the request must target the expected alternative sequence 45. Second, the observed state-change evidence must show that this selected sequence started from s4ener:Inactive.

S3-R2 – Requested start time satisfies the allowed time window

The requested start time must satisfy the selected sequence timing window. The requested start time must not be earlier than the earliest start time, the updated planned end must not exceed the latest end time, and the updated planned start must match the requested start time.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R2_RequestedStartTimeWithinAllowedWindow

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R2" ;

val:validationStep "Step1-InitialStateAndPreconditions" ;

rdfs:label "S3-R2 - Requested start time is within the allowed window" ;

sh:sparql [

sh:message "S3-R2 INCONCLUSIVE: requestedStartTime is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedStartTime ?requestedStart .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R2 INCONCLUSIVE: requestedPowerSequence is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R2 INCONCLUSIVE: selected sequence allowed time window is missing." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq

s4ener:hasEarliestStartTime ?earliestStart ;

s4ener:hasLatestEndTime ?latestEnd .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R2 INCONCLUSIVE: updated planned start or end time is missing for the selected sequence." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq

s4ener:hasStartTime ?updatedStart ;

s4ener:hasEndTime ?updatedEnd .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R2 FAIL: requested start time must be within the allowed window and must match the updated planned start time." ;

sh:select """

SELECT $this

WHERE {

$this

bito:requestedPowerSequence ?seq ;

bito:requestedStartTime ?requestedStart .

?seq

s4ener:hasEarliestStartTime ?earliestStart ;

s4ener:hasLatestEndTime ?latestEnd ;

s4ener:hasStartTime ?updatedStart ;

s4ener:hasEndTime ?updatedEnd .

FILTER (

?requestedStart < ?earliestStart ||

?updatedEnd > ?latestEnd ||

?updatedStart != ?requestedStart

)

}

""" ;

] .

This shape checks the lower timing bound, the upper timing bound, and the consistency between the request and the updated selected-sequence plan. The upper-bound check uses the mapped updated end time.

S3-R3 – Remote-control preconditions hold

The power profile must be remotely controllable, and the selected sequence must also be remotely controllable.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R3_RemoteControlPreconditions

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R3" ;

val:validationStep "Step1-InitialStateAndPreconditions" ;

rdfs:label "S3-R3 - Remote-control preconditions hold" ;

sh:sparql [

sh:message "S3-R3 INCONCLUSIVE: requestedPowerSequence is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R3 INCONCLUSIVE: nodeRemoteControllable declaration is missing from the power profile." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

<https://w3id.org/bito/example/flexible-start/device/000000000001/entity/0/power-profile>

s4ener:nodeRemoteControllable ?nodeRemoteControllable .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R3 FAIL: the power profile must be remotely controllable." ;

sh:select """

SELECT $this

WHERE {

<https://w3id.org/bito/example/flexible-start/device/000000000001/entity/0/power-profile>

s4ener:nodeRemoteControllable ?nodeRemoteControllable .

FILTER (?nodeRemoteControllable != true)

}

""" ;

] ;

sh:sparql [

sh:message "S3-R3 INCONCLUSIVE: sequenceRemoteControllable declaration is missing for the selected sequence." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq s4ener:sequenceRemoteControllable ?sequenceRemoteControllable .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R3 FAIL: the selected power sequence must be remotely controllable." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

?seq s4ener:sequenceRemoteControllable ?sequenceRemoteControllable .

FILTER (?sequenceRemoteControllable != true)

}

""" ;

] .

This shape checks the two remote-control preconditions required before the alternative sequence selection can be accepted. It uses s4ener:nodeRemoteControllable for the power profile and s4ener:sequenceRemoteControllable for the selected sequence.

S3-R4 – Reselection policy is respected

If the power profile declares supportsReselection=false, more than one accepted alternative-sequence selection request is not allowed.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R4_ReselectionPolicy

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R4" ;

val:validationStep "Step1-InitialStateAndPreconditions" ;

rdfs:label "S3-R4 - Reselection policy is respected" ;

sh:sparql [

sh:message "S3-R4 INCONCLUSIVE: supportsReselection declaration is missing from the power profile." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

<https://w3id.org/bito/example/flexible-start/device/000000000001/entity/0/power-profile>

s4ener:supportsReselection ?supportsReselection .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R4 FAIL: more than one accepted selection request is not allowed when supportsReselection is false." ;

sh:select """

SELECT $this

WHERE {

<https://w3id.org/bito/example/flexible-start/device/000000000001/entity/0/power-profile>

s4ener:supportsReselection false .

{

SELECT (COUNT(DISTINCT ?request) AS ?acceptedSelectionCount)

WHERE {

?request

a bito:ObservedEvent ;

bito:observedInteractionKind "alternative sequence selection request" ;

bito:acceptanceOutcome "accepted" .

}

}

FILTER (?acceptedSelectionCount > 1)

}

""" ;

] .

This shape checks the reselection policy. The counted request resources are explicitly constrained to bito:ObservedEvent, because interaction evidence is modeled as observed events.

S3-G1 – Acceptance gate

The observed selection request must be accepted before the state-transition checks are evaluated. S3-G1 is an implementation gate, not an additional behavioral rule.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_G1_AcceptanceGate

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-G1" ;

val:validationStep "Step2-AcceptedStateTransition" ;

rdfs:label "S3-G1 - Acceptance gate" ;

sh:sparql [

sh:message "S3-G1 INCONCLUSIVE: acceptanceOutcome is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:acceptanceOutcome ?outcome .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-G1 FAIL: the observed selection request must have acceptanceOutcome accepted." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome ?outcome .

FILTER (?outcome != "accepted")

}

""" ;

] .

This shape separates missing acceptance evidence from a negative behavioral result. Missing acceptanceOutcome is interpreted as inconclusive; a value different from "accepted" fails the acceptance gate.

S3-R5 – Selected sequence becomes scheduled after acceptance

After an accepted selection request, the selected sequence, here sequence 45, must become scheduled.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R5_SelectedSequenceBecomesScheduled

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R5" ;

val:validationStep "Step2-AcceptedStateTransition" ;

rdfs:label "S3-R5 - Selected sequence becomes scheduled" ;

sh:sparql [

sh:message "S3-R5 INCONCLUSIVE: requestedPowerSequence is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R5 INCONCLUSIVE: observed execution or trace evidence is missing for the accepted selection request." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R5 INCONCLUSIVE: selected sequence reference is missing from trace evidence." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

FILTER NOT EXISTS {

?traceEvidence bito:observedSelectedPowerSequence ?selectedSeq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R5 FAIL: trace evidence must identify sequence 45 as the selected sequence." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

?traceEvidence bito:observedSelectedPowerSequence ?selectedSeq .

FILTER (?selectedSeq != <https://w3id.org/bito/example/flexible-start/power-sequence/45>)

}

""" ;

] ;

sh:sparql [

sh:message "S3-R5 INCONCLUSIVE: observed state change for the selected sequence is missing." ;

sh:select """

SELECT $this

WHERE {

$this

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

?observedExecution bito:hasObservedEvent $this .

FILTER NOT EXISTS {

?observedExecution bito:hasObservedStateChange ?stateChange .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?seq ;

bito:hasObservedSourceState ?sourceState ;

bito:hasObservedTargetState ?targetState .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R5 FAIL: selected sequence must change from Inactive to Scheduled after acceptance." ;

sh:select """

SELECT $this

WHERE {

$this

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasObservedStateChange ?stateChange .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?seq ;

bito:hasObservedSourceState ?sourceState ;

bito:hasObservedTargetState ?targetState .

FILTER (

?sourceState != s4ener:Inactive ||

?targetState != s4ener:Scheduled

)

}

""" ;

] .

This shape checks the main Scenario 3 state transition. It uses bito:ObservedStateChange with bito:hasObservedSourceState and bito:hasObservedTargetState, so the transition is represented as a behavioral change, not as a static snapshot.

S3-R6 – Former preferred sequence becomes inactive after acceptance

After the accepted selection request, the former preferred sequence, here sequence 44, must become inactive.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R6_FormerPreferredSequenceBecomesInactive

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R6" ;

val:validationStep "Step2-AcceptedStateTransition" ;

rdfs:label "S3-R6 - Former preferred sequence becomes inactive" ;

sh:sparql [

sh:message "S3-R6 INCONCLUSIVE: observed execution or trace evidence is missing for the accepted selection request." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R6 INCONCLUSIVE: former preferred sequence reference is missing from trace evidence." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

FILTER NOT EXISTS {

?traceEvidence bito:observedFormerPreferredPowerSequence ?formerSeq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R6 FAIL: trace evidence must identify sequence 44 as the former preferred sequence." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

?traceEvidence bito:observedFormerPreferredPowerSequence ?formerSeq .

FILTER (?formerSeq != <https://w3id.org/bito/example/flexible-start/power-sequence/44>)

}

""" ;

] ;

sh:sparql [

sh:message "S3-R6 INCONCLUSIVE: observed state change for the former preferred sequence is missing." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

?traceEvidence bito:observedFormerPreferredPowerSequence ?formerSeq .

FILTER NOT EXISTS {

?observedExecution bito:hasObservedStateChange ?stateChange .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?formerSeq ;

bito:hasObservedSourceState ?sourceState ;

bito:hasObservedTargetState ?targetState .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R6 FAIL: former preferred sequence must change from Scheduled to Inactive after acceptance." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence ;

bito:hasObservedStateChange ?stateChange .

?traceEvidence bito:observedFormerPreferredPowerSequence ?formerSeq .

?stateChange

a bito:ObservedStateChange ;

bito:hasRelevantPowerSequence ?formerSeq ;

bito:hasObservedSourceState ?sourceState ;

bito:hasObservedTargetState ?targetState .

FILTER (

?sourceState != s4ener:Scheduled ||

?targetState != s4ener:Inactive

)

}

""" ;

] .

This shape checks the second Scenario 3 state transition. The former preferred sequence must move from s4ener:Scheduled to s4ener:Inactive. The trace evidence identifies which sequence was formerly preferred, and the observed state-change evidence validates its transition.

S3-R7 – Observed execution start shall not occur before the updated planned start

The observed execution start must not be earlier than the updated planned start time of the selected sequence.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix s4ener: <https://saref.etsi.org/saref4ener/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R7_ExecutionStartNotBeforeUpdatedPlan

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R7" ;

val:validationStep "Step3-ExecutionTimingAndOutcome" ;

rdfs:label "S3-R7 - Execution starts after updated planned start" ;

sh:sparql [

sh:message "S3-R7 INCONCLUSIVE: requestedPowerSequence is missing from the observed request event." ;

sh:select """

SELECT $this

WHERE {

FILTER NOT EXISTS {

$this bito:requestedPowerSequence ?seq .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R7 INCONCLUSIVE: observed execution is missing for the accepted selection request." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution bito:hasObservedEvent $this .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R7 INCONCLUSIVE: observed execution start time is missing." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution bito:hasObservedEvent $this .

FILTER NOT EXISTS {

?observedExecution bito:observedExecutionStart ?actualExecutionStart .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R7 INCONCLUSIVE: updated planned start time for the selected sequence is missing." ;

sh:select """

SELECT $this

WHERE {

$this bito:requestedPowerSequence ?seq .

FILTER NOT EXISTS {

?seq s4ener:hasStartTime ?updatedPlannedStart .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R7 FAIL: observed execution start must not be earlier than the updated planned start time." ;

sh:select """

SELECT $this

WHERE {

$this

bito:acceptanceOutcome "accepted" ;

bito:requestedPowerSequence ?seq .

?observedExecution

bito:hasObservedEvent $this ;

bito:observedExecutionStart ?actualExecutionStart .

?seq s4ener:hasStartTime ?updatedPlannedStart .

FILTER (?actualExecutionStart < ?updatedPlannedStart)

}

""" ;

] .

This shape checks temporal ordering between the updated selected-sequence plan and the observed execution. The updated planned start is read from s4ener:hasStartTime on the selected power sequence, and the actual execution start is read from the observed execution evidence.

S3-R8 – Observed run status is COMPLETED

The observed run status must be COMPLETED.

@prefix sh: <http://www.w3.org/ns/shacl#> .

@prefix val: <https://w3id.org/bito/example/flexible-start/validation/> .

@prefix bito: <https://w3id.org/bito#> .

@prefix ex: <https://w3id.org/bito/example/flexible-start/> .

@prefix dcterms: <http://purl.org/dc/terms/> .

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> .

val:Shape_S3_R8_RunStatusCompleted

a sh:NodeShape ;

sh:targetNode ex:ObservedEvent_Scenario3_SelectPowerSequence ;

dcterms:identifier "S3-R8" ;

val:validationStep "Step3-ExecutionTimingAndOutcome" ;

rdfs:label "S3-R8 - Observed run status is COMPLETED" ;

sh:sparql [

sh:message "S3-R8 INCONCLUSIVE: observed execution or trace evidence is missing for the accepted selection request." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

FILTER NOT EXISTS {

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R8 INCONCLUSIVE: observedRunStatus is missing from trace evidence." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

FILTER NOT EXISTS {

?traceEvidence bito:observedRunStatus ?runStatus .

}

}

""" ;

] ;

sh:sparql [

sh:message "S3-R8 FAIL: observedRunStatus must be COMPLETED." ;

sh:select """

SELECT $this

WHERE {

$this bito:acceptanceOutcome "accepted" .

?observedExecution

bito:hasObservedEvent $this ;

bito:hasTraceEvidence ?traceEvidence .

?traceEvidence bito:observedRunStatus ?runStatus .

FILTER (?runStatus != "COMPLETED")

}

""" ;

] .

This final outcome shape checks the observed run status from bito:TraceEvidence. It does not attach bito:observedRunStatus directly to bito:ObservedExecution, because the run-status evidence is represented as trace evidence.

# 9. Conclusion

This catalog keeps the device, plan, sequence, slot, state, and timing semantics grounded in reused ontologies and introduces bito: evidence-side properties only where the observed JSON evidence needs them. The result is a rule-by-rule validation basis that is traceable from dataset item to observed evidence item to ontology term to SHACL or SHACL-SPARQL validation logic.